# Quick Start Instructions

# PENNSYLVANIA Model 7500+ BENCH COUNTING SCALE

Manufactured in USA by :

Pennsylvania Scale Company – 1042 New Holland Avenue – Lancaster – Pennsylvania – 17601 – USA

For complete technical and full directions, please see enclosed manual or contact us at  www.pascale.com

| Step 1 | Step 2 | Step 3 | Step 4 | Continue… |
| --- | --- | --- | --- | --- |
| > PLACE Empty container on platform… | > PRESS Sample Set/Advance - Deducts weight of container and - Displays ‘ADD 10’* | > ADD Samples all at one time… Quantity added MUST match indicated quantity. | > COUNT | Same Part: Continue counting; if using a new container, place new container on platform and press ZERO at any time New Part: Go to Step 1… |
| Any suitable container can be used. | *Press again to advance through 4 preset sample sizes 10, 20, 50, 100 pieces | After ‘ – ‘ motion detect, scale will revert to COUNT mode | Add pieces to observe count. | It is not necessary to rezero the platform if using the SAME parts and the SAME container! |

Explanation of buttons and their function:

- ZERO – Rezeros the display for a new container weight; note the ‘ZERO’ indicator will illuminate to indicate zero condition.

- SAMPLE SET – Initiates the counting sequence by (a.) re-zeroing the platform and (b.) placing the scale in the ‘ADD’ mode for sampling.

- UNITS – Toggles the display and LED legends through the 2 standard weight units plus count:  lbs -> kgs (or g) -> COUNT

- PRINT – Causes information to be transferred by the digital interface to an optional printer or external display.

-2-

Quick Start Instructions – Helpful Hints for Best Results

- For best results, match the capacity of the scale to the relative size of parts to be counted, i.e. the lower the capacity of the scale, the higher the resolution and sensitivity.

- For small items, choose a larger sample size.  Generally a higher level of accuracy can be obtained during the critical sampling mode with a heavier sample.

- SAMPLE UPDATE ™ - This feature, software controlled, refines the average piece weight used for counting by refiguring based on a new whole quantity.  When active, the word ‘UPDATE’ will flash momentarily in the display.  If the rounding point is in a zone of uncertainty (as it would be if the part weights are inconsistent) the feature will not work.

- The piece weights of similar parts may vary due to material density differences, machine oil, manufacturing differences, lead lengths, moisture content in plastic items, different vendor sources, etc.  Since weigh counting depends on similar piece weights, count accuracy may be affected.  To improve results, we recommend using a larger sample quantity or using the Sample Update ™ feature.   To use Sample Update, after the initial sample, hand add small quantities of parts and observe the word ‘UPDATE’ flashing in the display momentarily.  The refined average piece weight will reflect a larger sample quantity without the necessity of hand counting a larger sample.

- OLOLOL – If the scale is overloaded greater than 105% of the rated capacity, it will display OLOLOL (overload!).  Remove items until a value is displayed and count the total in two drafts.  While very rugged, the load cell weight sensor can be damaged if overloaded or abused.  Use reasonable care to prevent damage to the load cell.

- It is OK if items are larger than the platform; keep the load centered as much as possible.

- To fill a container, bowl, carton, or other vessel press ‘ZERO’ to rezero the display and fill… it is not necessary to resample of continuing to count the same parts.

- Environmental factors (vibration, air currents, motion) can cause the display to appear unstable.  Pay attention to extra environmental influences.